| Risk Assessment Form |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Department: |  |  |  | RA Leader: |  |  |  | Approved by: |  |  |  |  |  | Reference No: |  |
| Process: |  |  |  | RA Member: |  |  |  |  |  |  |  |  |  |  |  |
| Activity / Location: |  |  |  |  |  |  |  | Name: |  |  |  |  |  |  |  |
| Original RA Date: |  |  |  |  |  |  |  | Designation: |  |  |  |  |  |  |  |
| This RA Review Date: |  |  |  |  |  |  |  | Date: |  |  |  |  |  |  |  |
| Next RA Review Date: |  |  |  |  |  |  |  | Rev No: |  |  |  |  |  |  |  |

| Ref | Sub-Activity | Hazard | Possible Injury | Affected Persons | Existing Risk Control | S | L | RPN | Additional Risk Control | S | L | RPN | Implementation Person | Due Date | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Commuting to worksite (airside driving and access) | Vehicle straying into aircraft movement areas due to driver unfamiliarity | Vehicle-aircraft collision, fatality, multiple severe injuries, major aircraft damage |  | 1. All drivers must attend Airside Safety Induction and Airport Operational & Safety (AOS) briefing (per Airport Ops Requirements) 2. Only drivers holding valid Airside Driving Permit (ADP) allowed to drive unescorted 3. Drivers without ADP using Temporary Entry Permit (TEP) vehicles must be escorted by an ADP holder (max 2 vehicles escorted) 4. New drivers orientated by experienced airside drivers on correct route 5. Daily toolbox briefing (multi-language) includes airside speed limit, route, and boundary 6. Updated aerodrome map showing designated routes in all vehicles 7. Instructions to stop in a safe location, activate hazard lights and call supervisor if unsure 8. Only perimeter road to be used to access Fire Station 3 9. Drivers to maintain safe distance from escort, never lose sight; stop and call for help if separated 10. Activated vehicle lights, warning flags, and high-visibility vests per airside policy | 5 | 2 | 10 | Regular route checks and update briefings; spot audits by Project Manager | 5 | 1 | 5 | Contractor Project Manager |  |  |
| 2 | Worksite set-up and access control | Unauthorised personnel or vehicles entering worksite—incursion into taxiway or operational areas | Risk of vehicle/person-aircraft collision resulting in fatalities or major injury; regulatory breach |  | 1. Cordon off work area with barriers and warning signage in line with Changi Airport Airside Works Procedure Manual 2. All access points monitored by staff 3. Briefing to all workers on restricted areas and reporting protocols 4. Close coordination with Airport Emergency Services (AES) Duty Officer before, during, and after work 5. Only personnel with valid permits (season pass or TEP with escort) allowed 6. Regular patrolling of perimeters during works 7. Controlled entry/exit log maintained | 5 | 2 | 10 | Installation of temporary CCTV at worksite entry; real-time perimeter monitoring where feasible | 5 | 1 | 5 | Site Supervisor |  |  |
| 3 | Surface cleaning and preparation (manual mopping, wiping, routine cleaning) | Slip and trip hazards from wet surfaces, cleaning solutions, or misplaced tools | Sprains, slips, falls, potential fractures, and soft tissue injuries |  | 1. Use of non-slip mats at entrances/exits 2. Immediate placement of warning signage on wet floors, as per SS 531 requirement for illumination 3. Cleaning only in cordoned, designated areas 4. Tools and materials stored clear of walk paths 5. Workers to wear slip-resistant, covered safety boots (SS 513) 6. Toolbox meeting includes slip/trip awareness | 3 | 3 | 9 | Spot checks for compliance and use of quick-dry/low-residue cleaning materials | 3 | 2 | 6 | Contractor Safety Personnel |  |  |
| 4 | Preparation and handling of anti-slip coating and treatment chemicals | Chemical splash/contact during mixing and handling | Skin or eye irritation, chemical burns, respiratory irritation |  | 1. PPE: Safety goggles, chemical-resistant gloves, appropriate masks (per SS 548 Part 2/3) 2. Chemical Safety Data Sheets (SDS) available onsite 3. Staff trained in chemical handling and PPE usage 4. Emergency eyewash and wash facility provided within 10m of handling area 5. No eating/drinking permitted in the work area 6. Toolbox talk covers chemical first-aid and PPE 7. Decanting only in well-ventilated areas | 3 | 2 | 6 | Display pictograms for PPE at entry points to chemical use zones | 3 | 1 | 3 | Site Supervisor |  |  |
| 5 | Application of anti-slip floor paint/chemical treatment (manual application—roller/brush) | Inhalation of volatile organic compounds (VOC) fumes from paints/treatments | Dizziness, headaches, respiratory discomfort, aggravation of asthma, possible long-term effects |  | 1. Selection of low-VOC water-based or low-toxicity coating wherever possible 2. Natural and mechanical ventilation enforced (doors/windows open, portable fans used) 3. Appropriate RPE (Respirator/half mask) for workers handling/working adjacent to application (per SS 548) 4. No application in confined, poorly ventilated enclosures 5. Maximum occupancy limits in work zone 6. Regular rest breaks | 3 | 2 | 6 | Install temporary exhaust fans in enclosed spaces; conduct periodic air monitoring with VOC meters | 3 | 1 | 3 | Contractor Safety Personnel |  |  |
| 6 | Handling and transfer of paint drums/chemical containers | Manual handling injuries (muscle strain, back injury); dropping drums causing spill | Back strain, muscle pull, foot injury, chemical splash, slip on spillage |  | 1. Use of trolleys or dollies for drum/container transport 2. Containers under 20kg per load per SS 569 ergonomic guidance 3. Two-man lifts for bulky/heavy drums 4. Only trained staff allowed to handle hazardous substances 5. Drum lids to be tightly sealed before moving 6. Clear path free of obstructions | 3 | 2 | 6 | Ergonomic training refreshers quarterly; first-aid kit checked for chemical splash dressings | 3 | 1 | 3 | Contractor Safety Personnel |  |  |
| 7 | Painting/floor treatment in vehicle bays (near operational fire vehicles and within airside zone) | Obstruction of fire vehicle access and AES emergency response routes | Delay in emergency response, risk to aircraft/passengers, indirect risk of fatalities |  | 1. Work only in cordoned off zones as agreed with AES 2. Direct communication line to AES Duty Officer at start and end of work each shift 3. Ensure all approach routes/gates to fire vehicles and taxiways remain clear 4. Only carry out works in approved phased portions; immediate removal of obstacles if requested by AES | 5 | 2 | 10 | Install temporary quick-release barricades on possible emergency vehicle lines; independent AES walkdown before work resumes each day | 5 | 1 | 5 | AES Liaison/Site Supervisor |  |  |
| 8 | Handling/mechanical agitation of paint or chemicals | Ignition of flammable vapours, fire hazard | Burns, smoke inhalation, property damage, risk to fire service personnel |  | 1. Only non-sparking, intrinsically safe tools used 2. No open flames or hot works unless hot work permit is obtained (per CAG and SCDF guidelines) 3. Paints and chemicals stored in original containers, labelled and kept away from electrical panels 4. Fire extinguisher and spill kit maintained at site 5. No smoking throughout airside per 'No Smoking' policy | 4 | 2 | 8 | Periodic fire patrol by AES during works; Safety signage for fire risk posted in designated areas | 4 | 1 | 4 | Contractor Safety Personnel |  |  |
| 9 | Work and material storage in vehicle bays (airside operations area) | Foreign Object Debris (FOD) introduction: unsecured tools/materials blown onto taxiway or apron | Aircraft engine ingestion, damage, potential fatality, FOD reporting incident |  | 1. All tools and materials securely fastened, checked before and after each shift per CAG FOD policy 2. FOD and tools check by supervisor at end of workday 3. Items in proper receptacles and secured before entering/leaving airside 4. Loose items checked and registered daily 5. Use of daily FOD inspection checklist (per CAG Airside Manual) | 5 | 2 | 10 | Random FOD checks by CAG Project Officer; staff reprimands for non-compliance | 5 | 1 | 5 | Site Supervisor |  |  |
| 10 | Chemical and general waste handling/disposal | Improper disposal causing environmental contamination or regulatory violation | Environmental pollution, legal penalty, potential exposure to airport wildlife |  | 1. Waste segregated according to type (chemical, general) and stored in covered bins 2. All chemical residues collected and removed daily by licensed waste contractor (NEA-licensed per SS 603) 3. No disposal in drains or airside areas 4. Proper documentation and manifests for special waste disposal 5. Daily waste removal by approved route (not via fire access roads) | 3 | 2 | 6 | Weekly waste audit and random checks by CAG Project Officer | 3 | 1 | 3 | Contractor Project Manager |  |  |
| 11 | Daily work phase start-up and briefing | Staff unfamiliar with airport/fire station emergency response requirements; inadequate safety briefing | Workers interfere with AES response, property damage, inadvertent triggering of alarm |  | 1. Daily toolbox briefings given in all relevant languages, with photographs and pictorial aids 2. Attendance recorded and verified each day 3. Supervisors to apply for system isolation if required 4. Coordination with AES to clarify zone boundaries and report times | 3 | 2 | 6 | Refresher briefings every major shift change or after incident; quiz/questionnaire validation | 3 | 1 | 3 | Site Supervisor |  |  |
| 12 | All site activities during periods of low/deteriorated visibility (e.g. haze, weather) | Reduced visibility resulting in collision between vehicles, pedestrians, or aircraft | Collision, property damage, potential fatality, injury from moving vehicles |  | 1. All vehicles must have operational headlights, warning lights, and personnel to use high-visibility vests per Changi Airport policy 2. Adequate lighting must be provided and all high-risk work to stop immediately under poor visibility 3. Close supervision and toolbox briefing each day 4. Suspension of works if visibility falls below airport operational minimum | 5 | 2 | 10 | Install temporary lighting towers and boundary indicators for works during marginal conditions | 5 | 1 | 5 | Project/Safety Manager |  |  |
| 13 | Floor painting/chemical application—enclosed space (toilet interiors) | Exposure to high concentrations of fumes due to insufficient ventilation | Acute respiratory symptoms, headaches, dizziness, loss of consciousness |  | 1. Work only with doors opened, portable fans deployed 2. Application limited to short, phased periods 3. Forced ventilation via exhaust fan if required 4. VOC/fume monitoring before and during work 5. Mandatory use of cartridge respirators for workers inside affected zone | 4 | 2 | 8 | Install fixed-point gas detector alarms for any prolonged use of solvent-based chemicals | 4 | 1 | 4 | Site Supervisor |  |  |
| 14 | Handling of tools and equipment (manual activities) | Musculoskeletal disorders from repetitive actions or awkward posture during floor work | Back pain, wrist/elbow strain, repetitive strain injury, chronic musculoskeletal disorder |  | 1. Use of long-handle rollers/brushes to minimise kneeling 2. Alternating work stations and jobs to allow micro-breaks 3. Staff trained in ergonomics and manual handling (SS 569) 4. Provision of kneepads, padded mats for floor work | 2 | 2 | 4 | Rotate tasks every 60 minutes; encourage micro-break exercises | 2 | 1 | 2 | Site Supervisor |  |  |
| 15 | Worksite hygiene and breaks (use of shared toilets/rest areas) | Transmission of infectious diseases due to inadequate hygiene practices | Stomach flu, respiratory tract infection, community-transmitted illness |  | 1. Handwashing facilities with soap and water provided 2. Toilets cleaned and disinfected daily 3. Staff reminded (toolbox brief) to observe hygiene 4. Masks and personal hygiene items to be provided/encouraged if illness is evident | 2 | 2 | 4 | Temperature and symptom screening if required during outbreaks | 2 | 1 | 2 | Site Supervisor |  |  |
| 16 | Ongoing operational activities (all work phases) | Unsecured items in vehicle bay fly into taxiway or runway (FOD risk) | Aircraft engine/tyre damage, potential accident or emergency |  | 1. All items in vehicle bays tied down before start and throughout shift 2. Supervisor to account for all items before/after work 3. FOD walk by Supervisor at end of the day 4. Daily FOD checklist submitted to CAG Project Officer | 5 | 2 | 10 | Random FOD audits by CAG; disciplinary action for non-compliance | 5 | 1 | 5 | Supervisor/Engineer/Safety personnel |  |  |
| 17 | Presence of food/edible waste in airside operational area | Wildlife (e.g., birds) attracted to worksite due to improper food waste disposal | Wildlife collides with aircraft, risk of engine ingestion, biodiversity and operational hazard |  | 1. Eating and littering strictly restricted to designated rest areas only 2. Daily toolbox reminder not to feed wildlife 3. Covered bins for general waste 4. Immediate waste removal at end of each day | 4 | 1 | 4 | Periodic wildlife patrol/inspection by airside operations | 4 | 1 | 4 | Contractor Project Manager |  |  |
| 18 | Handling and storage of equipment and tools | Noise from equipment or improper mechanical use causing hearing discomfort or distractions | Temporary hearing loss, distraction incidents, impaired communication |  | 1. Use of only low-noise mechanical and electric tools 2. All staff required to use hearing protection where noise >85 dBA 3. Briefing on noise hazards and reporting of issues | 2 | 2 | 4 | Provide signage on hearing protection; noise surveys quarterly | 2 | 1 | 2 | Site Supervisor |  |  |
| 19 | Works during emergency (fire/alarm/AES activation) | Workers and equipment obstructing emergency response or not evacuating promptly | Delayed emergency response, risk to life/property, injury to worker from fire vehicle movement |  | 1. All staff briefed on site-specific evacuation and emergency access route plans (CAG requirements) 2. Immediate stand-down and evacuation procedures rehearsed at start of project 3. At least 2 clear access paths for AES vehicles maintained at all times 4. Signage and marshals assigned during operational hours | 5 | 2 | 10 | Unplanned emergency drill at midpoint of project | 5 | 1 | 5 | AES Liaison/Site Supervisor |  |  |
| 20 | Housekeeping and worksite closeout each shift | Residual FOD, forgotten equipment, incomplete documentation | Potential operational incidents, regulatory non-compliance, injury from dropped object |  | 1. Daily end-of-shift checklist for FOD 2. Tools and equipment inventory signed off before and after work 3. Walkdown by supervisor, CAG, and AES as required 4. Documentation of any deficits or unresolved issues | 4 | 2 | 8 | Spot checks by CAG Project Officer | 4 | 1 | 4 | Shift Supervisor |  |  |
| 21 | Work with paint/chemical residues (wastewater control) | Uncontrolled runoff entering drains or airside movement area | Environmental contamination, pollution of operational area, wildlife hazard |  | 1. All liquid waste collected and transported by licensed contractor 2. Barriers placed around drains during mixing/application 3. No wash-downs into aprons/taxiways | 3 | 2 | 6 | Periodic drain checks and reporting to CAG/FMC | 3 | 1 | 3 | Site Supervisor |  |  |
| 22 | Initial and ongoing staff induction | Unfamiliarity with airside and fire station rules | Multiple (property, personal safety), possible regulatory fine for breach |  | 1. Mandatory attendance of Airside Safety Induction for all project personnel (including refresher for seasonal pass holders per CAG) 2. Induction record maintained and updated 3. Test/quiz part of induction prior to site access | 3 | 2 | 6 | Annual refresher for all staff; inclusion of latest CAG/circular updates in toolbox | 3 | 1 | 3 | Project Manager |  |  |
| 23 | Breakdown or leak of chemical containers | Chemical spill/accidental release on floor | Chemical burns, inhalation, slip hazard, environmental risk |  | 1. Spill containment kit at worksite 2. All staff trained in spill response using SDS 3. Notification of CAG AES and environmental agency immediately upon spill 4. Area immediately isolated and cleaned by trained personnel | 4 | 2 | 8 | Quarterly spill drill scenario conducted | 4 | 1 | 4 | Site Supervisor / Safety Personnel |  |  |
| 24 | Painting/chemical activities during operational hours | Distraction to fire station personnel (alarm fatigue, confusion in emergency) | Delayed response to fire/accident; missed alarm; injury/fatality as a result |  | 1. Prior consultation with AES before any shift 2. Communication protocol for immediate stand-down if alarm triggered 3. Works only during approved windows with cessation during alarms/alerts 4. On-site supervisor direct contact with AES duty personnel at all times | 5 | 2 | 10 | Alarm/evacuation drill at project midpoint | 5 | 1 | 5 | AES Liaison/Project Manager |  |  |
| 25 | General activity—team movement in vehicle bay and toilets | Collision between personnel, with doors, equipment, or operational vehicles | Minor injuries, cuts, contusions, musculoskeletal injuries |  | 1. Barricades/floor markings in line with CAG airside work procedures 2. Movement only along pre-assigned, clearly demarcated safe walkways 3. Staff briefed on high-traffic areas and mirror use on corners | 2 | 2 | 4 | Use of look-outs/flagmen if large teams involved | 2 | 1 | 2 | Site Supervisor |  |  |